PROJECT TITLE

7th Semester Project Report                                                                                                            submitted to

Techno India University, West Bengal

In partial fulfillment of the requirements

for the award of the degree of

Bachelor of Technology                                                                                                               in                                                                                                                                                               Computer Science & Engineering                                                                                         by

Shruti Mahato (221001001394)

Yuvaraj Dey (221001001388)

Prithweesh Banerjee (221001051001)

Susmoy Nath (221001001371)

Siddharth Gaur (221001001379)

Under the guidance of
SUPERVISOR(S) NAME

Dr. Abhra Mukherjee

Dr. Kalpita Dutta

Department of Computer Science & Engineering

TECHNO INDIA UNIVERSITY, WEST BENGAL
January 2026

© 2026, Techno India University. All rights reserved.

DECLARATION

I/We hereby declare that the project report entitled "Autonomous Drone Target Detection and Neutralization System" is an authentic record of our own work carried out at the Department of Computer Science and Engineering, Techno India University, West Bengal, during the 7th semester of the academic year 2025-2026 under the supervision of Dr. Abhra Mukherjee.

We further declare that the matter embodied in this project report has not been submitted by us for the award of any other degree or diploma of this or any other Institute/University.

Place: EM-4/1, Sector V, Bidhannagar, Kolkata, West Bengal 700091

Date:

[                                                  ] (Name: Shruti Mahato], Roll No: (221001001394)

[                                                  ] (Name: Yuvaraj Dey], Roll No: (221001001388)

[                                                  ] (Name: Prithweesh Banerjee], Roll No: (221001051001)

[                                                  ] (Name: Susmoy Nath], Roll No: (221001001371)

[                                                  ] (Name: Siddharth Gaur], Roll No: (221001001379)

ABSTRACT

The rapid technological advances in Unmanned Aerial Vehicle (UAV) have made it possible to create intelligent drone systems capable of carrying out autonomous tasks with very little human intervention. This work outlines the design and fabrication of an Autonomous Drone Target Detection and Neutralization System which, along with a quadcopter platform, shows the combination of autonomous navigation, embedded electronics, and precision actuation.

The main goal of the system is to locate a target automatically, get a lock on the target, follow the target in real time, and carry out a simulated precision payload release.

This system is equipped with onboard sensors, a vision, based detection method, and a microcontroller, based flight control structure for a stable and efficient autonomous operation. After a target is detected, the system permits manual or automatic locking, and then the drone goes into an autonomous follow mode.

A lightweight servo driven payload release mechanism is integrated to simulate a precision deployment operation when the drone is at an optimal position above the target.

Extensive testing and calibration are performed to confirm that the transitions between the different operational modes are carried out smoothly and the flight is stable. The project is a success in that it produces a working prototype that detects, tracks, navigates autonomously, and deploys payloads in a coordinated manner, thus laying down a solid framework for further upgrades in the field of autonomous UAV applications.

TABLE OF CONTENTS

LIST OF FIGURES

LIST OF TABLES

CHAPTER 1: INTRODUCTION

Smart UAS have reached the point where they can be completely automated to complete complex tasks with little to no human intervention. The earlier generations of UAVs required human intervention to make task decisions, but current UAVs can autonomously travel from point A to point B, detect objects, navigate obstacles and complete complex missions. These capabilities are achieved through the use of embedded electronics, on-board sensors, control algorithms, and mechanisms.

The Autonomous Drone Target Detection and Neutralization System was developed in order to show these capabilities at the prototype stage. The main objective was to manufacture a drone that can find, follow, and keep the location of a target, and drop a simulated pinpoint munition on the target. The design stresses the harmonious operation of flight dynamics, sensing, autonomous control, and mechanical actuation, and hence is ideal for controlled and small, area operations.

1.1            Drone Communication and Control

1. 1. 1 Hardware Configuration							           The drone platform is a quadcopter frame made from lightweight materials, to which are mounted brushless DC motors, Electronic Speed Controllers (ESCs), and a 3S Li, Po battery. The combination of these parts is capable of producing enough thrust, stability, and flight time for the drone to fly autonomously. In addition to the main components, there are structural supports such as spacers and landing pads, as well as cable management accessories that contribute to the mechanical strength of the frame and reduce vibrations during flight. A power distribution and switching system, that is not only reliable but also efficient, is designed to deliver power safely and under control to the various components that make up the drone.

1. 1. 2 Flight Control and Embedded System					        The drone control system is primarily a microcontroller, based flight controller which connects to different onboard sensors such as an Inertial Measurement Unit (IMU) and a barometric pressure sensor. The IMU is responsible for providing the flight controller with the orientation and angular rate of the drone continuously, whereas the barometric sensor is used for calculating the altitude of the drone during flight. With this data, the flight controller runs the necessary stabilization and control software routines to keep the drone in a state of balance, maintain its direction and altitude in both hand, guided as well as self, flying modes.

Manual flight control is the first to be carried out using the radio communication system (transmitter, and receiver) in order to check the drone's stability and the level of control prior to the activation of the autonomous functions.

1. 1. 3 Communication and Mode Switching
The unmanned aerial vehicle is controlled through a communication link using standard radio frequencies. This link gives possibilities for manual flight, system arming, and choice of flight mode. To enable flight without interruptions a unique changeover mechanism was installed. It makes easy change from manual flight to autonomous flight and vice versa. When autonomous flight mode is turned on, the aircraft's own controller takes over complete navigation and mission execution tasks.

1.2            Autonomous Functional Capabilities

1. 2. 1 Target Detection and Locking

This system is capable of figuring out a target by its own sensing capabilities. An onboard camera provides the visual content of the environment which is analyzed by means of computer vision techniques like color analysis, shape extraction, or marker detection. The moment a target is recognized, the system marks it visually and the operator gets the chance to lock the target manually or the system can do it automatically if the detection certainty is high enough.

Target locking is a feature with which the drone keeps its image acquisition component directed at one object continuously until the mission ends.

1. 2. 2 Autonomous Following and Navigation

Once a target is locked, the drone automatically changes to follow mode.

In follow mode, the drone keeps on figuring out the target's position relative to the camera frame. Using this info, the closed, loop control algorithms change the drone's yaw, forward, and lateral movement as well as altitude in order to keep a certain position relative to the target.

Thanks to this, the drone will be able to track both static and slow, moving objects within a small operational area without being dependent on GPS.

1. 2. 3 Precision Payload Deployment

The last stage of the operation is a mock precision drop. A mini servo motor is connected to a very light drop mechanism attached to the drone.

When the controller decides that the drone is best positioned above the locked target, the servo is operated to drop a small load, such as a metal piece. This motion will be a precision, hit or delivery operation simulation while the flight stability is still preserved

3            System Validation and Testing

To ensure reliable performance, the system undergoes extensive testing and refinement. Individual subsystems—such as detection, locking, autonomous following, and payload release—are tested separately and then evaluated in an integrated manner. Calibration of sensors and tuning of control parameters are performed to achieve smooth transitions between modes and stable autonomous behavior under varying conditions.